# Arduino-Based Capacitance Measurement – Early POC for Smart Tampon

## Objective

The purpose of this system is to measure the capacitance of a tampon using an Arduino-based setup and to transmit the data to a computer for analysis. In subsequent experiments, specific capacitance values will be mapped to saturation levels, where 10 mL of absorbed fluid will represent 100% saturation.

## System Overview

The measurement system is based on a modified version of the Arduino capacitance meter described in the following guide:
 🔗 Measure Capacitance with Arduino – Instructables

This system allows the estimation of low capacitance values in the picofarad (pF) range, suitable for detecting small changes in the tampon's capacitance due to fluid absorption.

## Components

| Component | Quantity | Description |
| --- | --- | --- |
| Arduino Uno (or Nano) | 1 | Main microcontroller unit |
| 1 MΩ Resistor | 1 | Used to form RC timing circuit |
| Test capacitor (tampon) | 1 | Two foil strips embedded along tampon sides |
| Breadboard + jump wires | – | For prototyping connections |
| USB cable | 1 | For Arduino-PC communication |
| Optional: PC UI interface | 1 | (e.g. Serial Monitor or Python GUI) |

## Measurement Principle

The Arduino code measures how long it takes for a known resistor (1 MΩ) to charge the unknown capacitor (i.e., the tampon) up to a threshold voltage. The time duration is proportional to the capacitance value.

Formula used:

Time to threshold ≈ R × C × ln(Vfinal/Vinitial)

The code then translates the time into an estimated capacitance value and sends it via serial communication to the connected computer.

## System Construction Steps

- Tampon Capacitor Preparation:

  - Attach two identical aluminum foil strips longitudinally along the tampon, acting as the capacitor plates.

  - Ensure strips do not touch each other.

  - Optionally insulate with a thin dielectric layer (e.g., plastic wrap or tape).

- Arduino Circuit Setup:

  - Connect one foil strip to a digital pin on the Arduino (e.g., D8).

  - Connect the second foil strip to GND.

  - Connect a 1 MΩ resistor between the digital pin and another pin configured for timing measurements (e.g., D9).

- Upload Arduino Code:

  - Use the code provided in the Instructables guide to calculate capacitance based on timing.

  - Modify the code if needed to improve sensitivity in the range of 5–50 pF.

- Connect to PC:

  - Use the Serial Monitor or a custom Python-based GUI to read and log capacitance data in real time.

## Next Phase – Calibration

In future experiments:

- A synthetic fluid simulating menstrual blood will be gradually added to the tampon to observe capacitance changes at increasing saturation levels.

- The composition of the simulated fluid is as follows:

### Simulated Menstrual Fluid Composition

- 100 mL warm water (non-boiling)

- 0.9 g table salt

- 30 mL honey

- 0.2 g cornstarch

- Red food coloring (for visual reference)

- The fluid will be applied incrementally using a syringe.

- Capacitance readings will be taken after each fluid increment.

- A mapping table will be constructed to correlate capacitance values to tampon saturation percentages:

  - 0% saturation = dry tampon

  - 100% saturation = 10 mL absorbed fluid

  - Intermediate levels will be estimated based on the data curve.

## Planned Enhancements

- Improved signal stability and noise reduction (e.g., averaging multiple readings).

- Integration with BLE module for wireless data transfer.

- Encapsulation of electrodes for medical-grade safety in future prototypes.

## Conclusion

This preliminary setup provides a low-cost and effective method for measuring the capacitance of a tampon and transmitting the results to a computer. It forms the basis for developing a real-time fluid monitoring system that will later be enhanced with calibrated saturation-level detection and potential mobile integration.